Case 1: Greenhouse Temperature Controller

Embedded Systems · UART, SPI and I2C · Tasks 2, 3 and 4 · Agriculture

Name: ______________________   ID: ____________   Date: ____________

| Task | Focus | Marks | Score | Signed |
| --- | --- | --- | --- | --- |
| Task 2 | UART / SPI / I2C protocols and functions | 100 |  | ☐ |
| Task 3 | Application layer, HAL, validation | 100 |  | ☐ |
| Task 4 | Layered architecture and Proteus design | 100 |  | ☐ |

Pass mark for each task: 50. Each task is assessed on its own. Grades per task: Distinction 85 to 100 · Merit 70 to 84 · Pass 50 to 69 · Not yet competent below 50.

# 1. Scenario

A greenhouse keeps its temperature between two limits. When it is too cold the heater turns ON. When it is too hot the fan turns ON. If the temperature rises above 35 °C a high-temperature alarm sounds. Staff set the two limits from a wall-mounted operator panel.

MCU A (operator panel) reads a DIP switch and buttons B1 and B2. It sends configuration by UART and the selected value by SPI. It never reads the sensor or switches an output. MCU B (controller) reads the temperature with an LM75 (I2C 0x48), stores MIN and MAX in a 24C02 EEPROM (I2C 0x50), shows data on an LCD (PCF8574, I2C 0x20), and switches the heater, fan, yellow LED and buzzer. It never reads the DIP switch or buttons directly.

| Protocol | Between | Carries | When |
| --- | --- | --- | --- |
| UART | MCU A to MCU B | Configuration only: MIN and MAX commands | On a button press |
| SPI | MCU A (master) to MCU B (slave) | Selected value only, one byte | About every 100 ms |
| I2C | MCU B to sensor | Measured value | Every loop |
| I2C | MCU B to 24C02 EEPROM (0x50) | MIN, MAX and a valid marker | Read at start, write on change |
| I2C | MCU B to LCD with PCF8574 (0x20) | Display text | When the text changes |

# 2. System rules

| Item | Value |
| --- | --- |
| Selected value (MCU A array) | {10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40} °C. DIP = 0101 is index 5, which is 20 °C. |
| First start | MIN = 18 °C, MAX = 28 °C |
| Heater | ON when T < MIN. OFF when T ≥ MIN + 1. |
| Fan | ON when T > MAX. OFF when T ≤ MAX − 1. |
| Gap rule | MAX must stay at least 2 °C above MIN |
| Alarm | Buzzer once each time T rises above 35 °C |
| Safe state on sensor fault | Heater OFF, fan OFF, yellow LED ON, LCD Sensor Fault |

## Programming rules (all tasks)

- Polling only. No interrupts and no timer peripherals. Use the status flags RXC0, SPIF and TWINT.

- _delay_ms is allowed for button debounce, the 100 ms SPI spacing and the buzzer beep.

- Use C and the ATmega328P registers. No Arduino libraries (Wire, SPI, Serial, EEPROM, LiquidCrystal_I2C).

- MCU A reads inputs and sends data. MCU B decides and acts.

# 3. Data formats and device details

## 3.1 UART frames (configuration only)

9600 baud, 8 data bits, no parity, 1 stop bit. B1 sends the MIN command and B2 sends the MAX command. Format: command, one space, two digits, CR, LF. Capital letters only. MCU B treats LF as the end of the line, ignores CR, ignores any line longer than 19 characters, and sends no reply.

| Command | Meaning | Value range | Frame bytes |
| --- | --- | --- | --- |
| SETMIN | Set MIN | 10 to 40 | 11 |
| SETMAX | Set MAX | 15 to 50 | 11 |

| Example frame | Hex bytes |
| --- | --- |
| SETMIN 20 + CR LF | 53 45 54 4D 49 4E 20 32 30 0D 0A |
| SETMAX 30 + CR LF | 53 45 54 4D 41 58 20 33 30 0D 0A |

## 3.2 SPI frame (selected value only)

One byte, the selected value as a binary number. Mode 0, MSB first, clock F_CPU / 16, SS low during the transfer, about every 100 ms. MCU B ignores a byte outside 10 to 40 and shows a valid one on the LCD as Sel:xx.

| DIP | Index | Selected value | SPI byte |
| --- | --- | --- | --- |
| 0000 | 0 | 10 | 0x0A |
| 0101 | 5 | 20 | 0x14 |
| 1010 | 10 | 30 | 0x1E |
| 1111 | 15 | 40 | 0x28 |

## 3.3 I2C sensor

I2C runs at 100 kHz with 4.7 kΩ pull-ups on SDA (PC4) and SCL (PC5). The LM75 is read with the pointer register 0x00. Bus bytes are 0x90 (write) and 0x91 (read). Use the MSB as whole degrees (signed). The LSB bit 7 is the 0.5 °C bit and is not used in this case study.

START | 0x90 | ACK | 0x00 | ACK | REPEATED START | 0x91 | ACK |

MSB (ACK) | LSB (NACK) | STOP

| MSB | LSB | Value used (whole °C) |
| --- | --- | --- |
| 0x19 | 0x00 | 25 |
| 0x19 | 0x80 | 25 (25.5 truncated) |
| 0x28 | 0x00 | 40 |

Three failed sensor reads in a row put the system in its safe state. One good read clears the fault.

## 3.4 EEPROM and LCD

EEPROM 24C02 (bus bytes 0xA0 write, 0xA1 read): MIN at 0x00, MAX at 0x01, valid marker 0xA5 at 0x02. Wait 5 ms after each write. Write only when a command is accepted. At power-up, if the marker is missing, save the defaults.

LCD 16×2 with PCF8574 (0x20), 4-bit mode: P7..P4 = D7..D4, P3 = backlight, P2 = E, P1 = RW, P0 = RS. Layout:

| Situation | Line 1 | Line 2 |
| --- | --- | --- |
| Normal | Temp:25 C Sel:20 | H:OFF F:OFF |
| Sensor fault | Sensor Fault | blank |

# Task 2: UART / SPI / I2C protocols and functions (100 marks)

Outcome: the student can write and use the three serial protocols to move data between devices. What you write: the protocol functions for both MCUs. Header files with the function names are provided.

| ID | Requirement |
| --- | --- |
| U1 | Configure UART at 9600 8N1. Write functions to send a byte, send a string and receive a byte. |
| U2 | MCU A sends the MIN command on B1 and the MAX command on B2, once per press, in the frame format of Section 3.1. |
| U3 | MCU B reads characters until LF and ignores CR. It ignores a line longer than 19 characters and sends no reply. |
| S1 | Configure SPI master (MCU A) and slave (MCU B) in mode 0, MSB first, F_CPU / 16. |
| S2 | MCU A pulls SS low, sends one byte, and releases SS. MCU B detects the byte with the SPIF flag. |
| I1 | Write I2C (TWI) functions at 100 kHz: start, repeated start, write byte, read byte with ACK, read byte with NACK, stop. Return an error if a device does not ACK. |
| I2 | Read the sensor as shown in Section 3.3 (LM75 pointer read). |
| I3 | Write one byte to the EEPROM and read it back with a random read. Wait 5 ms after the write. |
| I4 | Send one byte to the LCD expander (0x20) and confirm the ACK. |

Evidence: source code, plus a screenshot from the Proteus UART terminal, SPI debugger and I2C debugger for each frame.

| Marking | Marks |
| --- | --- |
| UART functions (U1 to U3) | 30 |
| SPI master and slave (S1, S2) | 25 |
| I2C functions and device reads or writes (I1 to I4) | 35 |
| Polling only, comments, no Arduino libraries | 10 |
| Total | 100 |

Examiner comments: ______________________________________________________________

# Task 3: Application layer, HAL, validation (100 marks)

Outcome: the student can build a working application on top of a hardware abstraction layer and prove that it meets the requirements. What you write: the HAL, the application and the validation. If Task 2 is not finished, the examiner supplies a reference protocol library so Task 3 can still be assessed.

### HAL (write these)

| Module | Functions | Uses |
| --- | --- | --- |
| Inputs (MCU A) | dip_read(), button_pressed() | GPIO |
| Outputs (MCU B) | Heater and fan set function(s), led_fault_set(), buzzer_beep() | GPIO |
| Sensor | temp_read() returns whole °C or an error | I2C |
| Memory | ee_save(), ee_load() for MIN, MAX and the marker 0xA5 | I2C |
| Display | lcd_init(), lcd_print(), lcd_clear() | I2C |

### Application requirements

| ID | Requirement |
| --- | --- |
| A1 | MCU A: look up the selected value from the DIP value (array in Section 2). Send the MIN command on B1 and the MAX command on B2 (debounced, once per press). Send the selected value by SPI every 100 ms. |
| A2 | MCU B: ignore a UART line with lower case, a value out of range, or a gap smaller than the gap rule in Section 2. |
| A3 | MCU B: ignore an SPI byte outside 10 to 40 and show a valid one as Sel:xx. |
| A4 | Load MIN and MAX at power-up. If the marker is missing, save the defaults. Save only when a command is accepted. |
| A5 | Apply the control rule in Section 2, including hysteresis and "keep the last state" where it is stated. |
| A6 | After 3 failed sensor reads in a row, enter the safe state in Section 2. A good read clears it. |
| A7 | Apply the alarm rule in Section 2 (once per event, no repeat while the condition stays). |
| A8 | Show the LCD layout in Section 3.4. |

### Validation: required tests

Write at least 12 test cases in the table format below, run them, and record the actual result and pass or fail. Include these checks (outputs start OFF) and add your own if needed.

| Test | Input | Expected result |
| --- | --- | --- |
| Heater | MIN = 20. Temperature 19, 20, 21 | ON, ON, OFF |
| Fan | MAX = 30. Temperature 31, 30, 29 | ON, ON, OFF |
| Good command | DIP = 0101, press B1 | MIN becomes 20 |
| Lower case | setmin 20 | Ignored |
| Out of range | SETMIN 55 | Ignored |
| Gap rule | SETMAX 21 when MIN = 20 | Ignored |
| SPI good | DIP = 1010 | Byte 0x1E, LCD Sel:30 |
| SPI bad | Byte 0x32 (50) | Ignored |
| High alarm | Temperature 34, 35, 36, 37, 30, 36 | Beep at 36, none at 37, beep at second 36 |
| EEPROM | Set MIN = 20, power cycle | MIN is still 20 |
| Sensor fault | Remove the LM75, wait 3 reads | Sensor Fault, heater and fan OFF, LED ON |
| Recovery | Reconnect the LM75 | Normal operation returns |

Your test record format:

| Test ID | Requirement | Input | Expected result | Actual result | Pass / Fail |
| --- | --- | --- | --- | --- | --- |

| Marking | Marks |
| --- | --- |
| HAL modules work and hide the protocol details | 30 |
| Application behavior (A1 to A8) | 40 |
| Test cases: coverage, negative and boundary tests | 15 |
| Test results: actual results recorded and a pass or fail summary | 15 |
| Total | 100 |

Penalties: application code touches registers directly (−10). MCU A reads the sensor or MCU B reads the DIP or buttons (−10 each).

Examiner comments: ______________________________________________________________

# Task 4: Layered architecture and Proteus design (100 marks)

Outcome: the student can design the system in layers and build and simulate it in Proteus.

### Deliverables

- Layered architecture diagram for MCU A and MCU B: Application layer, HAL layer, Protocol layer (UART, SPI, I2C) and hardware (registers and pins). Show every module, the allowed call from each layer to the layer directly below, and the direction of data.

- Context diagram: the system as one block with every outside item: operator, DIP switch, buttons, greenhouse temperature, the output device(s), buzzer, yellow LED and LCD. Label each arrow with the data and the protocol.

- Proteus schematic with net names, pin numbers, a virtual terminal, an SPI debugger and an I2C debugger. Parts for this case are listed below.

- Simulation evidence: screenshots that show one UART frame, one SPI byte, one sensor read, and the LCD in normal mode.

### Proteus parts for this case

- MCU A: ATmega328P, 4-bit DIP switch, buttons B1 and B2, UART TX, SPI master pins

- MCU B: ATmega328P, UART RX, SPI slave pins, I2C bus with 4.7 kΩ pull-ups, 24C02 (0x50), PCF8574 LCD (0x20), yellow LED, buzzer

- LM75 at 0x48 (set A0 to A2 to ground)

- Heater and fan as LEDs or relays, yellow LED, buzzer

- A temperature source: change the LM75 temperature in Proteus

| Marking | Marks |
| --- | --- |
| Layered architecture diagram: layers, modules, calls, data direction | 30 |
| Context diagram: all items and labeled protocol arrows | 15 |
| Proteus schematic: correct wiring, addresses, pull-ups, labels | 35 |
| Simulation evidence: runs with your firmware, frames and LCD shown | 20 |
| Total | 100 |

Penalty: a layer in the diagram calls a layer that is not directly below it (−10).

Examiner comments: ______________________________________________________________